Kit # 18 Instructions

Collection of Viral Specimens

The WSLH virus specimen collection kit (kit # 18) is designed for collecting and shipping specimens for virus PCR testing.    Alternatively, other manufacturer’s virus transport medium (VTM) are also acceptable for submitting virus specimens. If virus transport medium is unavailable, consult WSLH Customer Service (phone 608-224-4272 or 800-862-1013).

Vials of virus transport medium may be stored at room temperature. However, vials should be refrigerated immediately after specimen collection.  Do NOT use if vials show leakage, or if transport medium appears contaminated (i.e. cloudy) or purple-colored.

**DESCRIPTION OF WSLH KIT #18**

| KIT CONTENTS | COMMENTS |
| --- | --- |
| Vial of viral transport medium (M4-RT) | Use for swab, tissue, scrapings, body fluid; refrigerate immediately after collection. |
| Absorbent wrap | Wrap specimen container in absorbent material before shipping. |
| 95 kPa Pressure bag | Enclose wrapped specimen container in bag; insert form in outer sleeve. |
| White plastic cold pack, styrofoam mailer and carton (MRV) | Cold pack should be frozen solid before mailing specimens. Place frozen cold pack in kit before shipping. |
| Mailing label and UN3373 label | Attach to mailer prior to shipping. |
| Swabs- included with media | Use Flocked wire swab for nasopharyngeal, urethral specimens. Use Flocked plastic shaft swab for throat or lesion specimens. |
| Instructions For Collecting & Shipping Viral Specimens |  |
| (If Requested) “CDD Requisition Form (A)” test requisition forms |  |

NOTE:  WSLH Virus Transport Medium contains antibiotics and SHOULD NOT be given to patients for collection of gargles or washes.

SPECIMEN HANDLING

All specimens for viral culture or PCR should be refrigerated (at 2 – 80C) immediately after collection and kept cold during transit, as many viruses are heat labile.  Specimens should be transported to the WSLH without delay.  If specimen transport is delayed >72 hours, freeze and ship on dry ice. Testing of specimens may be delayed or refused if improperly collected, improperly transported, or submitted with insufficient patient and clinical information.  Note that at least two patient identifiers must be provided on specimen containers and requisition forms, in order to meet regulatory requirements.

COLLECTION OF VIRAL SPECIMENS

Specimens should be obtained as soon as possible after illness onset, when the greatest amount of virus is present; the chance of virus recovery decrease as the illness progresses.

Polyester-, or Dacron-tipped swabs with plastic or aluminum shafts are recommended.  (Do NOT use calcium alginate-tipped swabs, as calcium alginate swabs are toxic to some viruses.  Avoid using cotton-tipped or wood-shafted swabs.)

Cut the shafts of swabs so that caps of specimen containers can be securely tightened.

Standard aseptic specimen collection procedures should be followed, along with the additional guidelines included in these instructions.

COLLECTION OF VIRAL SPECIMENS (continued)

For specific virus specimen collection recommendations, please refer to the WSLH reference manual at http://www.slh.wisc.edu/outreach-data/wslhSearch.php

General Specimen Collection Guidelines

| SPECIMEN SOURCE | COLLECTION PROCEDURE | ADDITIONAL COMMENTS |
| --- | --- | --- |
| CSF | Submit 2 - 5 ml in sterile, empty, screw-capped container. | Serum must accompany CSF when requesting CSF antibody titers. |
| Eye | Place swabs or scrapings in VTM vial. |  |
| Stool | Place 10-20 grams (walnut-sized portion) in Carey-Blair stool collection kit or clean, empty, screw-capped container. | Rectal swab is acceptable, but not preferred. |
| Throat / Nasopharyngeal | Collect both an NP swab AND a throat swab and place in the same vial of VTM |  |
| Vesicular Lesion | Vigorously collect swab or scraping from base of a NEW vesicle and place in vial of VTM. | Vesicle fluid (with swab or scraping) is preferred, if vesicle is intact tissue. |
| Tissue | Collect each tissue with separate, sterile instruments and place in separate VTM vials. | If tissue does not fit in vial, place in sterile, screw-capped container |
| Urine | Submit 5 ml in sterile, empty, screw-capped container. |  |
| Other Fluid (including Broncho-Alveolar Lavage or Brushing) | Submit 2 - 5 ml in sterile, empty, screw-capped container. |  |
| Other Swabs | Place swab in VTM vial. |  |

Label each specimen container with patient's name, date of birth, source of specimen, and date of collection.  Refrigerate specimens immediately after collection.

Packaging and Transport

- Secure all tube or vial caps with tape or parafilm.

- Place bagged specimens into a pressure bag; seal bag securely.

- Include cushioning material as needed to prevent breakage.

- Complete a WSLH Requisition Form and place in outer pocket of bag.

- Place sample in mailer along with frozen cold pack; seal mailer with tape.

- Place WSLH address label and Category B/UN3373 label on mailer.

- Attach label with the name, telephone number, and address of a person who has knowledge of the contents of the shipment.

- Ship in accordance with current shipping regulations.

- Select the most rapid method of transportation available (24 hour delivery is preferred).  Specimens should be shipped to arrive at the laboratory before 4:30 p.m. Monday through Friday or before 12:30 p.m. on Saturday.  Packages arriving after these hours or on Sundays may be subjected to unfavorable temperatures which could compromise specimen quality.

Contact Information

WSLH:  www.slh.wisc.edu

Customer Service:  (800) 862-1013 or (608) 224-4272

Clinical Orders:  (800) 862-1088 or (608) 224-4275

Reference Manual:  http://www.slh.wisc.edu/wslhApps/RefMan/wslhSearch.php